# OCTADECAFLUOROOCTANE    CAS # 307346

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE    2.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECANE    CAS # 593453

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .0001000 mm Hg @ 20 °C

FLASH POINT  330 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  33.7 ppm

DOE Short Term Exposure Limit  33.7 ppm

DOE Ceiling Limit 173 ppm

Immediately Dangerous to Life and Health   750 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECANOL (1-)    CAS # 112925

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 20000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  365 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2.5 ppm

DOE Short Term Exposure Limit  7.5 ppm

DOE Ceiling Limit 50 ppm

Immediately Dangerous to Life and Health   750 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECENE (1-)    CAS # 112889

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  309.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECYLAMINE    CAS # 124301

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2395.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  298.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle    and store under nitrogen\. Incompatible

Materials: Absorbs carbon dioxide from air\. SPECIAL REQUIREMENTS Handle and

store under inert gas. Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECYLTRIMETHOXYSILANE    CAS # 3069429

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  284 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Silicon oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Moisture

sensitive. Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADECYLTRIMETHYLAMMONIUM CHLORIDE    CAS # 112038

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1729.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: Harmful if absorbed through skin.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

Multiple Routes: Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22 36/37/38

Risk Statements: Harmful in contact with skin and if swallowed.

Irritating to eyes, respiratory system and skin.

S: 26 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADIENE (1,7-)    CAS # 3710303

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 19700.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50  36000.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  50 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. Do    not breathe

vapor. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Keep away from heat, sparks, and open flame. Store in a cool dry place. Store

under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xn

Indication of Danger: Highly Flammable. Harmful.

R: 11 65 52/53

Risk Statements: Highly flammable. Harmful: may cause lung

damage if swallowed. Harmful to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 16 61 62

Safety Statements: Keep away from sources of ignition - no

smoking. Avoid release to the environment. Refer to special

instructions/safety data sheets. If swallowed, do not induce

vomiting: seek medical advice immediately and show this

container or label.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTADIYNE (1,7-)    CAS # 871841

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  74 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 16 26 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTAETHYLPORPHINE    CAS # 2683821

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTAHYDROBINAPHTHALENEDIOL (5,5,6,6',7,7:8,8:1,1':    CAS # 65355002

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

-

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.    -

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANE    CAS # 111659

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 1 - LD50         mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION RISK INDEX   <1  - LC50 ********

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Can cause CNS depression. Narcotic effect.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   11.0 mm Hg @ 20 °C

FLASH POINT  55.4 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Handle and store under nitrogen\.

SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xn N

Indication of Danger: Highly Flammable. Harmful. Dangerous for

the environment.

R: 11 38 50/53 65 67

Risk Statements: Highly flammable. Irritating to skin. Very

toxic to aquatic organisms, may cause long-term adverse effects

in the aquatic environment. Harmful: may cause lung damage if

swallowed. Vapors may cause drowsiness and dizziness.

S: 9 16 29 33 60 61 62

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Do not empty

into drains. Take precautionary measures against static

discharges. This material and its container must be disposed of

as hazardous waste. Avoid release to the environment. Refer to

special instructions/safety data sheets. If swallowed, do not

induce vomiting: seek medical advice immediately and show this

container or label.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 300 ppm

OSHA Short Term Exposure Limit  375 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 300 ppm

Immediately Dangerous to Life and Health  1000 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  300 ppm

DOE Short Term Exposure Limit  300 ppm

DOE Ceiling Limit 385 ppm

Immediately Dangerous to Life and Health   1000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANEDIOL (1,8-)    CAS # 629414

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  248 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Reducing agents, Chloroformates, Oxidizing  agents, Acid

anhydrides, Acid chlorides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANESULFONATE (1-)    CAS # 221562

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE  SULFUR OXIDES

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANESULFONIC ACID SODIUM (1-)    CAS # 5324845

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING    and clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

Risk Statements: Toxic if swallowed.

S: 36/37/39 45

Safety Statements: Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANOIC ACID    CAS # 124072

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2395.0 mg/Kg

INHALATION RISK INDEX   1.1 - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .210 mm Hg @ 20 °C

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate

foam\. Protective Equipment: Wear self-contained breathing apparatus and

protective clothing to prevent contact with skin and eyes.    Specific

Hazard(s): Emits toxic fumes un

TOXIC EMISSIONS WHEN BURNED: Hazardous Polymerization: Will

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on STORAGE: Keep tightly

closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

R: 34

Risk Statements: Causes burns.

S: 26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

Indication of Danger: Corrosive.

Risk Statements: Causes burns.

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  40 mg/m3

DOE Short Term Exposure Limit  125 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANOL (1-)    CAS # 111875

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3200.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Will not occur

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Can cause CNS depression. Exposure can cause: Nausea, headache,

and vomiting. Narcotic effect. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .110 mm Hg @ 20 °C

FLASH POINT  176 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/38

Risk Statements: Irritating to eyes and skin.

S: 26 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  50 mg/m3

DOE Short Term Exposure Limit  50 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   350 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANOL (2-)    CAS # 123966

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3200.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Nausea, dizziness, and headache.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .190 mm Hg @ 20 °C

FLASH POINT  160 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/38

Risk Statements: Irritating to eyes and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  35 mg/m3

DOE Ceiling Limit 100 mg/m3

Immediately Dangerous to Life and Health   100 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANONE (2-)    CAS # 111137

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3089.0 mg/Kg

INHALATION RISK INDEX   1.4 - LC50  11157.8

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .777 mm Hg @ 20 °C

FLASH POINT  132.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing  agents, Strong

bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21

Risk Statements: Harmful in contact with skin.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  60 mg/m3

DOE Short Term Exposure Limit  200 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTANOYL CHLORIDE    CAS # 111648   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   2   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    2.5 mm Hg @ 20 °C

FLASH POINT  179.6 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Water, Alcohols, Oxidizing agents, Strong  bases.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place\. SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTENE (1-)    CAS # 111660

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

ACUTE TOXICTY RISK INDEX 2 - LD50 10000.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.0 - LC50  36872.3

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Will not occur

SIGNS AND SYMPTOMS OF EXPOSURE

Headache, dizziness, drowsiness, incoordination, slowed reaction

time, slurred speech, giddiness and unconsciousness. Exposure

can cause: Prolonged or repeated contact with skin can cause

defatting and dermatitis. Contact with eyes can cause redness,

tearing, and blurred vision. Ingestion may cause

gastrointestinal irritation. Aspiration or inhalation may cause

chemical pneumonitis.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   17.9 mm Hg @ 20 °C

FLASH POINT  70 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   CORROSIVE

INCOMPATIBILITIES:Oxidizing agents, Acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate    foam. Water can be

applied as a spray or fog and if properly    applied is capable of

extinguishing the fi

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-Xn-N

Indication of Danger: Highly Flammable. Harmful. Dangerous for

the environment.

R: 11-51/53-65

Risk Statements: Highly flammable. Toxic to aquatic organisms,

may cause long-term adverse effects in the aquatic environment.

Harmful: may cause lung damage if swallowed.

S: 16-60-62

Safety Statements: Keep away from sources of ignition - no

smoking. This material and its container must be disposed of as

hazardous waste. If swallowed, do not induce vomiting: seek

medical advice immediately and show this container or label.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  40 ppm

DOE Short Term Exposure Limit  40 ppm

DOE Ceiling Limit 800 ppm

Immediately Dangerous to Life and Health   2000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTOPAMINE HYDROCHLORIDE    CAS # 4502141

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

SYMPTOMS OF OVEREXPOSURE THAT MAY OCCUR ARE: REFLEX BRADYCARDIA,

UNDESIRABLE ELEVATED BLOOD PRESSURE WITH HEADACHE AND VOMITING ALONG

WITH PALPITATIONS.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTOPAMINE HYDROCHLORIDE (DL-)    CAS # 770058

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of overexposure that may occur are: reflex bradycardia,

undesirable elevated blood pressure with headache, and vomiting

along with palpitations. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYL ALDEHYDE    CAS # 124130

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  5630.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.4 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

IRRITATION DATA

skin

Rabbit   500 mg

24H

Remarks: Mild irritation effect

Eyes

Rabbit

100 mg

Remarks: Mild irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE  680.0 mm Hg @ 20 °C

FLASH POINT  125.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing agents, Strong

bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 10

Risk Statements: Flammable.

S: 16

Safety Statements: Keep away from sources of ignition - no

smoking.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYL-AGAROSE    CAS # 68652095

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYL-SEPHAROSE    CAS # 69106601

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 10

Risk Statements: Flammable.

S: 16

Safety Statements: Keep away from sources of ignition - no

smoking.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYLAMINE    CAS # 111864

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   F   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4 - LD50    50.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   2.8 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE***** mm Hg @ 20 °C

FLASH POINT  140 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid  chlorides,

Acid anhydrides.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .4 mg/m3

DOE Short Term Exposure Limit  1.25 mg/m3

DOE Ceiling Limit 7.5 mg/m3

Immediately Dangerous to Life and Health   40 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYLGLUCOPYRANOSIDE (B,D-)    CAS # 29836268

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYLMETHOXY CINNAMATE (4-)    CAS # 5466773

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE

REACTIVE PROPERTIES

MINOR AMOUNTS OF ACID CAUSE ACROLEIN TO POYLMERIZE WITH RELEASE OF

HEAT.BENZYL ALCOHOL CONTAINING ACIDIC CONSTITUENTS AND DISSOLVED IRON WAS

FOUND TOPOLYMERIZE WITH A RAPID TEMPERATURE INCREASE WHEN HEATED IN EXCESS

OF100C.(AMINES, PYRIDINE, AND ALKALI HYDROXIDES ACT AS INHIBITORS AND

PREVENTPOLYMERIZATION.) REACT VIGOROUSLY WITH LITHIUM ALUMINUM HYDRIDE.

HIGHCONCENTRATIONS OF ACID REACT EXPLOSIVELY WITH NICKEL NITRIDE AND

HEAT.INITIATE A FAST DECOMPOSITION OF SODIUM OZONATE. REACTS WITH

THORIUMPHOSPHIDE TO RELEASE SPONTANEOUSLY FLAMMABLE PHSOPHINE. REACTS

VIOLENTLY WITHTRI-ISO-BUTYL ALUMINUM.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYLPHENOL (TERT,4-)    CAS # 140669

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 2 - LD50  4600.0 mg/Kg

INHALATION RISK INDEX   1.4 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE    1.1 mm Hg @ 20 °C

FLASH POINT  293 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21 38 41

Risk Statements: Harmful in contact with skin. Irritating to

skin. Risk of serious damage to eyes.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OCTYLTRIMETHYLAMMONIUM CHLORIDE    CAS # 10108868

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3400.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas Ammonia

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OIL RED EGN    CAS # 4477796

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OIL RED O    CAS # 1320065

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

Liver. Blood. Eyes. Kidneys. Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  156.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING    and clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25 22

Safety Statements: Avoid contact with skin and eyes. Do not

breathe dust.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OKADAIC ACID    CAS # 78111178

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. Toxic if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Smooth muscle.

SIGNS AND SYMPTOMS OF EXPOSURE

Gastrointestinal disturbances. Exposure can cause: Tumor

promoter.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25-38

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Irritating to skin.

S: 26-36/37-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OLEIC ACID    CAS # 112801

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX .8 - LD50 74000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .110 mm Hg @ 20 °C

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air and light sensitive. Heat sensitive.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .015 mg/m3

DOE Short Term Exposure Limit  .05 mg/m3

DOE Ceiling Limit .4 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OLEIC ACID SODIUM    CAS # 143191

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

ACUTE TOXICTY RISK INDEX 2 - LD50  5140.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE Store at -20░C

SPECIAL REQUIREMENTS

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OLIVE OIL    CAS # 8001250

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

UNITED STATES REGULATORY INFORMATION

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OMEPRAZOLE    CAS # 73590586

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2210.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

G.I. System.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea. Diarrhea. Gastrointestinal

disturbances. Headache. Dizziness. Skin rash. Increased liver

enzymes. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORANGE G SODIUM    CAS # 1936158

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORANGE II    CAS # 633965

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORANGE IV    CAS # 554734

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

STORAGE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORCEIN    CAS # 1400620

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

STORAGE

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORCINOL MONOHYDRATE    CAS # 6153395

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.8 - LD50   844.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Depending on the intensity and duration of exposure, effects may

vary from mild irritation to severe destruction of tissue.

Prolonged contact can cause: Damage to the eyes.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air and light sensitive. Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORNITHINE HYDROCHLORIDE (D-)    CAS # 16682125

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# ORNITHINE HYDROCHLORIDE (L-)    CAS # 3184132

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OROTIC ACID ANHYDROUS    CAS # 65861

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  6451.6 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OSMIUM    CAS # 7440042

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   4   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SENSITIZATION

Sensitization: Causes dermatitis.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxygen, Halogens, Phosphorus.

FIRE EXTINGUISHER:  Dry chemical powder.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. STORAGE: Keep tightly closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 37/38 41

Risk Statements: Highly flammable. Irritating to respiratory

system and skin. Risk of serious damage to eyes.

S: 16 26 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .25 mg/m3

DOE Short Term Exposure Limit  .75 mg/m3

DOE Ceiling Limit 6 mg/m3

Immediately Dangerous to Life and Health   30 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OSMIUM TETROXIDE    CAS # 20816120

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   H   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 3 - LD50   522.6 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.7 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be fatal if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: May be fatal if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be fatal if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes. Central nervous system. Male reproductive system. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Symptoms may be delayed. Exposure can cause: A

greenish or blackish discoloration of the skin may result from

direct contact. May cause an asthmatic-like bronchitis. Exposure

may cause excessive lacrimation and a sensation of seeing halos

or colored rings around lights. Inhalation may result in spasm,

inflammation and edema of the larynxand bronchi, chemical

pneumonitis, and pulmonary edema. Material is extremely

destructive to tissue of the mucous membranes and upper

respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE    7.0 mm Hg @ 20 °C

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Contact with hydrochloric acid will cause  formation of

poisonous chlorine gas, Finely powdered metals,    Organic materials, Strong

reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sublimes easily and releases a  poisonous and

irritating vapor

Store at 2-8°C

Keep tightly closed. Keep away from combustible materials, heat, sparks, and

open flame. Store in a

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame. Store in a cool

dry    place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 26/27/28-34

Risk Statements: Very toxic by inhalation, in contact with skin

and if swallowed. Causes burns.

S: 7/9-26-45

Safety Statements: Keep container tightly closed and in

well-ventilated place. In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

Immediately Dangerous to Life and Health  1 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .0027 ppm

DOE Short Term Exposure Limit  .0027 ppm

DOE Ceiling Limit .0096 ppm

Immediately Dangerous to Life and Health   1.34 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OUABAIN    CAS # 11018896

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ACUTE EFFECTS

MAY CAUSE SKIN IRRITATION.

MAY BE HARMFUL IF ABSORBED THROUGH THE SKIN.

MAY CAUSE EYE IRRITATION.

MATERIAL MAY BE IRRITATING TO MUCOUS MEMBRANES AND UPPER

RESPIRATORY TRACT.

TOXIC IF INHALED.

MAY BE FATAL IF SWALLOWED.

CHRONIC EFFECTS

TARGET ORGAN(S):

HEART

RTECS #: RN3850000

OUABAIN OCTAHYDRATE

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.      CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: DECOMPOSITION PRODUCTS CARB

REACTIVE PROPERTIES

-

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALACETIC ACID    CAS # 328427

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be fatal if absorbed through skin.

Inhalation: May be fatal if inhaled. Material may be irritating

to mucous membranes and upper respiratory tract.

Ingestion: May be fatal if swallowed.

Nerves.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

TOXIC EMISSIONS WHEN BURNED: Mercury/mercury oxides HAZARDOUS POLYMERIZATION

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID    CAS # 6153566

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.1 - LD50   375.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Severe eye irritant.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Nerves. Blood. Eyes.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases Avoid contact with metals., Acid  chlorides, Alkali

metals

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22

Risk Statements: Harmful in contact with skin and if swallowed.

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

Immediately Dangerous to Life and Health  500 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  2 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID ANHYDROUS    CAS # 144627

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7500.0 mg/Kg

NEUROTOXIC - RISK INDEX   2.0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Nerves. Blood. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Alkali metals Avoid contact with metals.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Immediately Dangerous to Life and Health  500 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  2 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID DIAMIDE    CAS # 471465

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

toxicological properties have not been thoroughly investigated.

IRRITATION DATA

Rabbit

100 mg

24H

Remarks: Moderate irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, STORAGE: Keep

tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26-36/37/39-45

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 40 mg/m3

Immediately Dangerous to Life and Health   200 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID DIAMMONIUM    CAS # 1113388

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Nerves. Blood. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin. To the

best of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated. Inhalation may

result in spasm, inflammation and edema of the larynxand

bronchi, chemical pneumonitis, and pulmonary edema. Symptoms of

exposure may include burning sensation, coughing, wheezing,

laryngitis, shortness of breath, headache, nausea, and vomiting.

Damage to the kidneys.

IRRITATION DATA

Eyes

Remarks: Severe irritation effect

skin

Remarks: Severe irritation effect

Inhalation

Remarks: Severe irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Ammonia

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22

Risk Statements: Harmful in contact with skin and if swallowed.

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .15 mg/m3

DOE Short Term Exposure Limit  .5 mg/m3

DOE Ceiling Limit 4 mg/m3

Immediately Dangerous to Life and Health   20 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID DILITHIUM    CAS # 553913

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: Harmful if absorbed through skin.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Nerves. Blood. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Large doses of lithium ion have caused dizziness and

prostration, and can cause kidney damage if sodium intake is

limited. Dehydration, weight loss, dermatological effects, and

thyroid disturbances have been reported. Central nervous system

effects that include slurred speech, blurred vision, sensory

loss, ataxia, and convulsions may occur. Diarrhea, vomiting, and

neuromuscular effects such as tremor, clonus, and hyperactive

reflexes may occur as a result of repeated exposure to lithium

ion. To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Lithium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 21/22 34 63

Risk Statements: Harmful in contact with skin and if swallowed.

Causes burns. Possible risk of harm to the unborn child.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALIC ACID DIPOTASSIUM MONOHYDRATE    CAS # 6487485

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Nerves. Blood. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22

Risk Statements: Harmful in contact with skin and if swallowed.

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.5 mg/m3

DOE Short Term Exposure Limit  25 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXALYL CHLORIDE    CAS # 79378   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 3 - LD50         mg/Kg

INHALATION RISK INDEX   <1  - LC50   9532.7

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Lachrymator. Causes severe burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. Toxic if

inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE  150.0 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Bases, Oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Store under

inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 20 29 34

Risk Statements: Reacts violently with water. Harmful by

inhalation. Contact with water liberates toxic gas. Causes burns.

S: 26 36/37/39 43 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of fire, never use water. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 ppm

DOE Short Term Exposure Limit  5 ppm

DOE Ceiling Limit 40 ppm

Immediately Dangerous to Life and Health   200 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXAZOLE    CAS # 288426

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  66 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 41

Risk Statements: Highly flammable. Risk of serious damage to

eyes.

S: 16 26 36

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXAZOLIDONE (2-)    CAS # 497256

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36 43

Risk Statements: Harmful if swallowed. Irritating to eyes. May

cause sensitization by skin contact.

S: 26 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXIBENDAZOLE    CAS # 20559551

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXIRANE    CAS # 93356753

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXO TEMPO (4-)    CAS # 2896700

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1364.5 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXOLINIC ACID    CAS # 14698294

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50   525.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, STORAGE: Keep tightly

closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXYGEN    CAS # 7782447   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Breathing high concentrations of oxygen (greater than 75 mole

percent) causes symptoms of hyperoxia which include cramps,

nausea, dizziness, hypothermia, ambylopia, respiratory

difficulties, bradycardia, fainting spells and convulsions

capable of leading to death. May be harmful.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

VAPOR PRESSURE******* mm Hg @ 20 °C

Oxidizing Agent

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

i - CorrosiveorNon-Toxic Gas

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Phosphorus, Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Use water spray or fog nozzle to keep cylinder cool. Move   cylinder away

from fire if there is no risk.

REACTIVE PROPERTIES

HANDLING: Do not breathe gas. Do not get in eyes, on skin, on clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

combustible   materials, heat, sparks, and open flame. Store in a cool dry

place. Cylinder temperature should not exceed 125░F (52░C)\. SPECIAL

REQUIREMENTS   Contents under pressure.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O

Indication of Danger: Oxidizing.

R: 8

Risk Statements: Contact with combustible material may cause

fire.

S: 17

Safety Statements: Keep away from combustible material.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  35000 ppm

DOE Short Term Exposure Limit  65000 ppm

DOE Ceiling Limit 230000 ppm

Immediately Dangerous to Life and Health   400000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXYLTETRAMETHYL PIPERIDINE (2,2,6,6-)    CAS # 2564832   HAZARDOUS CHEMICAL

OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Ears. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: ON

REACTIVE PROPERTIES

User Exposure: Do not breathe vapor. Avoid contact with eyes, skin, and

clothing. STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXYTETRACYCLINE DIHYDRATE    CAS # 6153646

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Teeth. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXYTETRACYCLINE HYDROCHLORIDE    CAS # 2058460

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Some dithiocarbamates have been reported to have teratogenic

and/or carcinogenic potential. (1974 Evaluations of Some

261 to 263. Geneva,1975; IARC vol 7, 1974, vol 12, 1976) and to

affect male reproductive capacity (target organ toxicity center

reproductive information file). Exposure to thiocarbamates or

thiurams and the intake of even small amounts of ethyl alcohol

low blood pressure. Sensitization to alcohol may last as long as

6 to 14 days after exposure to these compounds. A serious toxic

interaction has been observed in rats fed tetraethylthiuram and

then exposed to vapors of 1,2-dibromoethane. (NIOSH Current

Intelligence Bulletin 23, US Dept. Hew April 11, 1978). NIOSH

chemicals structurally related to tetraethylthiuram and other

halogenated hydrocarbons.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases.

FIRE EXTINGUISHER: appropriate foam.

TOXIC EMISSIONS WHEN BURNED: ODUCTS  Nitrogen oxides Hydrogen chloride gas

HAZARDOUS POLYMERIZATION

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, STORAGE: Keep

tightly closed. Store in a cool dry place. Store under nitrogen\. SPECIAL

REQUIREMENTS  Store under inert gas. Light sensitive. Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 38 41

Risk Statements: Harmful if swallowed. Irritating to skin. Risk

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# OXYTOCIN    CAS # 50566

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.4 - LD50    20.5 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Cardiovascular system. Uterus.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 60

Risk Statements: May impair fertility.

S: 45

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.